# Vanier College – Advanced Networks
Fall 2026
Teacher: Tiago Bortoletto Vaz

# Subnetting: Multi-Department Network

Name: Willkerdy Elveus          Date: 09/17/2026

## Before you start: build it like a professional

Set up your topology the way a network engineer would document it, before assigning any IP addresses:

•  Rename devices with meaningful hostnames (R-HQ, R-Remote, SW-Sales, PC-Sales-01) instead of Packet Tracer defaults - click the name label, or use Config > Global Settings > Display Name.

•  Label departments on the canvas (Place Note tool) if hostnames alone aren't clear enough.

•  Use Copper Straight-Through for PC-to-switch and switch-to-router links, and Fiber for the router-to-router WAN link if using SFP fiber modules.

•  Any switch model works here - no VLANs or advanced config needed.

•  Recommended router: Cisco 2911 (3 onboard GigabitEthernet ports covers the 3 departments). For the 4th interface (WAN link), power the router off, add an HWIC-1GE-SFP in an empty HWIC slot, seat a GLC-LH-SMD in its SFP cage, then power back on.

## Scenario

You are given 192.168.10.0/24 to divide among three department LANs on R1, one remote-site LAN on R2, and a point-to-point fiber link between R1 and R2. Departments have different host counts, so size each subnet to fit - largest first.

## Given requirements

This is typically all the information you'd get from a client or manager: departments, rough headcounts, which sites connect. No addresses or masks - turning this into an addressing plan is your job in Step 1.

| Segment | Hosts needed | Connects to |
| --- | --- | --- |
| Sales LAN | up to 50 | R1 (R-HQ) |
| Engineering LAN | up to 20 | R1 (R-HQ) |
| Management LAN | up to 10 | R1 (R-HQ) |
| Remote LAN | up to 5 | R2 (R-Remote) |
| R1-R2 WAN link | 2 (point-to-point) | R1 & R2 |

## Step 1: Subnet the address block

Fill in every column for each segment, largest requirement first.

| Segment | Network address | CIDR | Subnet mask | First usable host | Last usable host | Broadcast address | Usable hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sales | 192.168.10.0 | /26 | 255.255.255.192 | 10.1 | 10.62 | 10.63 | 62 |
| Engineering | 192.168.10.64 | /27 | 255.255.255.224 | 10.65 | 10.94 | 10.95 | 30 |
| Management | 192.168.10.96 | /28 | 255.255.255.240 | 10.97 | 10.110 | 10.111 | 14 |
| Remote | 192.168.10.112 | /29 | 255.255.255.248 | 10.113 | 10.118 | 10.119 | 6 |
| WAN link (R1-R2) | 192.168.10.120 | /30 | 255.255.255.252 | 10.121 | 10.122 | 10.123 | 2 |

## Step 2: Assign router interface addresses

No default-gateway column here - a router doesn't need one. A gateway is for a device that can't route and hands off anything unfamiliar to one trusted device. A router keeps its own routing table instead, and adds each configured interface's subnet to it automatically as a 'connected' route.

| Router interface | IP address | Subnet mask |
| --- | --- | --- |
| R1 (R-HQ) - Sales interface | 192.168.10.1 | 255.255.255.192 |
| R1 (R-HQ) - Engineering interface | 192.168.10.65 | 255.255.255.224 |
| R1 (R-HQ) - Management interface | 192.168.10.97 | 255.255.255.240 |
| R1 (R-HQ) - WAN interface | 192.168.10.121 | 255.255.255.252 |
| R2 (R-Remote) - Remote interface | 192.168.10.113 | 255.255.255.248 |
| R2 (R-Remote) - WAN interface |  |  |

## Step 3: Assign PC addresses

PCs can't route, so they need a default gateway: the router interface on their own subnet. Sales PCs use R1's Sales interface (192.168.10.1); Remote PCs use R2's Remote interface (192.168.10.113); and so on.

| Device | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| Sales - PC1 |  |  |  |
| Sales - PC2 |  |  |  |
| Engineering - PC1 |  |  |  |
| Engineering - PC2 |  |  |  |
| Management - PC1 |  |  |  |
| Management - PC2 |  |  |  |
| Remote - PC1 |  |  |  |
| Remote - PC2 |  |  |  |

## Checkpoint: test before you fix it

Configure everything above, then ping a Remote PC from a Sales PC. It will fail. Before reading on, write down why - think about what Step 2 said about how a router learns its routes.

## Step 4: Static routes needed

A router only learns about its own directly connected networks automatically - it has no way to discover anything further away, even across a link to another router. Since no dynamic routing protocol is running here, each router needs to be manually told: 'this network isn't mine, but send matching traffic to this next hop.' That's what fixes the checkpoint failure.

| Router | Destination network | Mask | Next hop |
| --- | --- | --- | --- |

## Step 5: Fundamental questions

You should be able to answer these well enough to rebuild this lab from scratch, with different numbers, unaided.

1. Why is it useful to divide a single large address block into several smaller subnets instead of one flat network?

2. Why size the largest requirement first when subnetting for several different-sized needs?

3. What is the difference between a network address and a broadcast address in the same subnet?

4. Why does a router interface not need a default gateway, while a PC does?

5. A ping from Sales to Remote failed even though every device had correct addressing. Why?